LIMO ISA Specification

CS3520 — Computer Organisation and Architecture

Team: Architects

Repository: saintchigos/limo-architects

Milestone: M1

## A1. Processor Style

LIMO is a 32-bit, load–store, RISC-V-derived instruction set architecture.

| Property | LIMO Decision |
| --- | --- |
| Architecture class | RISC (load–store only) |
| Word size | 32 bits |
| Memory model | Byte-addressed |
| Byte order | Little-endian |
| Instruction size | Fixed 32 bits |
| Instruction formats | R, I, S, B |
| Word alignment | Word-aligned (addresses must be multiples of 4) |
| Assembly language | Sesotho-language mnemonics |

Load–store: only bala (load) and boloka (store) access data memory. All other instructions operate exclusively on registers.

Little-endian: for a 32-bit value stored at address A, the least significant byte is at A, the most significant byte at A+3. This matches RV32I and ensures straightforward cross-checking against Ripes.

Word-alignment: all instruction fetches and word memory accesses must use addresses divisible by 4. The simulator reports a misalignment error otherwise. This falls naturally from fixed 32-bit instructions: if the first instruction is at address 0, subsequent instructions are at 4, 8, 12, and so on.

Omitted formats: U-type and J-type are not included. LIMO's 12-bit I-type immediate is sufficient for all targeted programs. The absence of J-type means long-range unconditional branches are not supported; this is documented as the primary structural limitation in A6(f). Any departure from RV32I field layouts is justified here.

## A2. Register File

LIMO has 32 general-purpose registers (r0–r31), each 32 bits wide. Register indices are 5 bits (2⁵ = 32).

### Programmer-visible special-purpose registers

| Number | Assembly name | Sesotho meaning | Role |
| --- | --- | --- | --- |
| PC (separate) | sebali_tsamaiso | program runner | Architectural PC; separate from r0–r31 and not addressable through GPR fields |
| r0 | noto | zero | Hardwired to 0; all writes are silently discarded |
| r1 | khutla_aterese | return address | Stores return address on procedure calls |
| r2 | ts'upiso_phaello | stack pointer | Points to the current top of the call stack |
| r3 | ts'upiso_kakaretso | global pointer | Points to the global data region |

PC numbering note: sebali_tsamaiso is a separate architectural special-purpose register, not one of the numbered GPRs. It is therefore shown as PC (separate) rather than being assigned a GPR index. The 5-bit register fields address only r0–r31.

### General-purpose registers

| Numbers | Group | Sesotho name | Role |
| --- | --- | --- | --- |
| r4–r7 | Argument registers | li_khang | Function arguments / caller-saved |
| r8–r9 | Return value registers | li_phetho | Function return values |
| r10–r15 | Temporary registers | li_nakoana | Not preserved across calls |
| r16–r31 | Saved registers | li_boloka | Callee-saved; preserved across calls |

Note on `ts'upiso_phaello`: the apostrophe in ts' is a Sesotho digraph representing a distinct phoneme. See A4 for how the assembler handles apostrophes in identifiers.

## A3. Instruction Set

LIMO implements 12 instructions across four classes. Its conditional branches are lekana (BEQ), hali_lekane (BNE) and kholo_ho (BGE). No multiply/divide, floating-point, CSRs or exceptions are included.

### Arithmetic — 3 instructions (including 1 immediate)

| # | Mnemonic | Syntax | Operation | Format | RV32I equivalent |
| --- | --- | --- | --- | --- | --- |
| 1 | kopanya | kopanya rd, rs1, rs2 | rd = rs1 + rs2 | R | ADD |
| 2 | tlosa | tlosa rd, rs1, rs2 | rd = rs1 − rs2 | R | SUB |
| 3 | kopanya_hang | kopanya_hang rd, rs1, imm | rd = rs1 + imm | I | ADDI |

### Logic — 3 instructions (including 1 NOT)

| # | Mnemonic | Syntax | Operation | Format | RV32I equivalent |
| --- | --- | --- | --- | --- | --- |
| 4 | le | le rd, rs1, rs2 | rd = rs1 AND rs2 | R | AND |
| 5 | kapa | kapa rd, rs1, rs2 | rd = rs1 OR rs2 | R | OR |
| 6 | hase | hase rd, rs1 | rd = ~rs1 (bitwise NOT) | R | — (pseudo: XORI rd, rs1, −1) |

Note on `hase`: RISC-V has no dedicated NOT instruction; it uses XORI rd, rs1, -1. LIMO provides it as a real R-type instruction. The assembler sets rs2 = r0 (noto) and the processor ignores the rs2 field.

### Shift — 1 instruction

| # | Mnemonic | Syntax | Operation | Format | RV32I equivalent |
| --- | --- | --- | --- | --- | --- |
| 7 | checha_hang_leqeleng | checha_hang_leqeleng rd, rs1, shamt | rd = rs1 << shamt | I | SLLI |

Only the lower 5 bits of the immediate (shamt) are used; a 32-bit register can only be shifted 0–31 positions.

### Memory — 2 instructions

| # | Mnemonic | Syntax | Operation | Format | RV32I equivalent |
| --- | --- | --- | --- | --- | --- |
| 8 | bala | bala rd, imm(rs1) | rd = Memory[rs1 + imm] | I | LW |
| 9 | boloka | boloka rs2, imm(rs1) | Memory[rs1 + imm] = rs2 | S | SW |

### Branches — 3 instructions

| # | Mnemonic | Syntax | Operation | Format | RV32I equivalent |
| --- | --- | --- | --- | --- | --- |
| 10 | lekana | lekana rs1, rs2, label | if rs1 == rs2: PC = label | B | BEQ |
| 11 | kholo_ho | kholo_ho rs1, rs2, label | if rs1 ≥ rs2: PC = label | B | BGE |
| 12 | hali_lekane | hali_lekane rs1, rs2, label | if rs1 != rs2: PC = label | B | BNE |

Note on `hali_lekane`: this is LIMO's Sesotho mnemonic for BNE (branch if not equal). It uses the standard RV32I B-type funct3 = 001; the branch is taken when rs1 != rs2.

## A4. Sesotho Assembly

### Mnemonic Glossary

| Mnemonic | Sesotho meaning | Operation | RV32I equivalent |
| --- | --- | --- | --- |
| kopanya | combine / add together | rd = rs1 + rs2 | ADD |
| tlosa | remove / take away | rd = rs1 − rs2 | SUB |
| kopanya_hang | add right now / add immediately | rd = rs1 + imm | ADDI |
| le | and (conjunction) | rd = rs1 AND rs2 | AND |
| kapa | or (disjunction) | rd = rs1 OR rs2 | OR |
| hase | it is not / negate | rd = ~rs1 | — |
| checha_hang_leqeleng | shift immediately to the left | rd = rs1 << shamt | SLLI |
| bala | read / load | rd = Memory[rs1 + imm] | LW |
| boloka | save / store | Memory[rs1 + imm] = rs2 | SW |
| lekana | equal / balanced | branch if rs1 == rs2 | BEQ |
| kholo_ho | greater than or equal | branch if rs1 ≥ rs2 | BGE |
| hali_lekane | not equal | branch if rs1 != rs2 | BNE |

### Assembly syntax

# This is a comment
label:
    mnemonic rd, rs1, rs2       # R-type
    mnemonic rd, rs1, imm       # I-type
    mnemonic rs2, imm(rs1)      # S-type
    mnemonic rs1, rs2, label    # B-type

Comments begin with #. Labels end with :. Immediates may be decimal or hexadecimal (0x prefix).

### Apostrophe handling

Sesotho orthography includes digraphs ts' and ch', where the apostrophe denotes a distinct ejective consonant, not a delimiter. These appear in register names such as ts'upiso_phaello and ts'upiso_kakaretso.

Tokenisation rule: an identifier begins with a letter and continues consuming characters that are letters, digits, underscores (_), or apostrophes ('). Tokenisation stops at whitespace, a comma, a parenthesis, or end of line. This means ts'upiso_phaello is a single token.

Comment delimiter: the # character begins a comment, not the apostrophe. There is no ambiguity.

Example: bala r1, 0(ts'upiso_phaello) tokenises as: bala, r1, ,, 0, (, ts'upiso_phaello, ).

## A5. Encoding and Specification

### Instruction formats

All formats use the same field layout as their RV32I counterparts. The field positions are familiar, but LIMO's custom opcode values mean LIMO machine code is not directly executable as RV32I.

R-type

31      25 24    20 19    15 14  12 11     7 6      0
+----------+--------+--------+------+--------+--------+
|  funct7  |  rs2   |  rs1   | fn3  |   rd   | opcode |
|  7 bits  | 5 bits | 5 bits |3 bits| 5 bits | 7 bits |
+----------+--------+--------+------+--------+--------+

I-type

31            20 19    15 14  12 11     7 6      0
+----------------+--------+------+--------+--------+
|   imm[11:0]   |  rs1   | fn3  |   rd   | opcode |
|   12 bits     | 5 bits |3 bits| 5 bits | 7 bits |
+----------------+--------+------+--------+--------+

S-type

31      25 24    20 19    15 14  12 11     7 6      0
+----------+--------+--------+------+--------+--------+
|imm[11:5] |  rs2   |  rs1   | fn3  |imm[4:0]| opcode |
|  7 bits  | 5 bits | 5 bits |3 bits| 5 bits | 7 bits |
+----------+--------+--------+------+--------+--------+

B-type

31 30      25 24    20 19    15 14  12 11    8 7 6      0
+--+----------+--------+--------+------+--------+-+--------+
|12|imm[10:5] |  rs2   |  rs1   | fn3  |imm[4:1]|11|opcode|
|  | 6 bits  | 5 bits | 5 bits |3 bits| 4 bits | |7 bits |
+--+----------+--------+--------+------+--------+-+--------+

The B-type immediate encodes a 13-bit signed byte offset with bit 0 implicitly 0 (word-aligned targets). This gives a branch reach of ±4 KB.

### Opcode assignments

| Format | Opcode (binary) | Opcode (hex) |
| --- | --- | --- |
| R | 1001101 | 0x4D |
| I | 1001001 | 0x49 |
| S | 0101101 | 0x2D |
| B | 1001110 | 0x4E |

### funct3 and funct7 assignments

R-type (opcode = 1001101)

| Instruction | funct3 | funct7 |
| --- | --- | --- |
| kopanya | 000 | 0000000 |
| tlosa | 000 | 0000001 |
| le | 001 | 0000010 |
| kapa | 101 | 0000011 |
| hase | 010 | 0000100 |

I-type (opcode = 1001001)

| Instruction | funct3 |
| --- | --- |
| bala | 000 |
| kopanya_hang | 100 |
| checha_hang_leqeleng | 101 |

S-type (opcode = 0101101)

| Instruction | funct3 |
| --- | --- |
| boloka | 000 |

B-type (opcode = 1001110)

| Instruction | funct3 |
| --- | --- |
| lekana (BEQ) | 000 |
| hali_lekane (BNE) | 001 |
| kholo_ho (BGE) | 101 |

### Rationale for encoding assignments

| Encoding choice | Rationale |
| --- | --- |
| Separate opcode for each format class (R, I, S, B) | Lets the decoder identify the instruction format before interpreting the remaining fields. The chosen 7-bit values are LIMO-specific and do not claim binary compatibility with RV32I machine code. |
| 5-bit register fields | Supports all 32 GPRs (r0–r31). It costs 15 bits in an R-type instruction for rs1, rs2 and rd, but avoids the register pressure of a smaller register file. |
| R-type funct3/funct7 combinations | The pair distinguishes LIMO's register-register operations that share the R-type opcode. kopanya and tlosa share funct3=000 and are distinguished by funct7; logic operations have their own combinations. |
| I-type funct3 | Distinguishes bala, kopanya_hang and checha_hang_leqeleng under the I-type opcode. Shift amount uses only the low five immediate bits because a 32-bit value has shift positions 0–31. |
| S-type funct3=000 | Identifies the word-store operation boloka; the immediate is split across the standard S-type fields to preserve a signed 12-bit offset. |
| B-type funct3 | Uses standard RV32I values for lekana/BEQ (000), hali_lekane/BNE (001) and kholo_ho/BGE (101) so the branch meaning is explicit and consistent with the documented equivalents. |
| 12-bit I/S immediates and 13-bit B offset | Keeps formats compact. I/S offsets are signed 12-bit values; B-type offsets encode an even byte offset with bit 0 implicit, allowing a wider branch reach than a plain 12-bit immediate. |
| No U/J formats | Reduces the first implementation's scope. The trade-off is that LIMO cannot directly encode long-range jumps; A6(f) identifies this limitation and recommends a future J-type instruction. |

### Immediate ranges

| Format | Immediate bits | Signed range |
| --- | --- | --- |
| I | 12-bit signed | −2048 to +2047 |
| S | 12-bit signed | −2048 to +2047 |
| B | 13-bit signed, bit 0 = 0 | −4096 to +4094 (multiples of 2) |

### Hand-encoded examples (one per format)

#### R-type: kopanya r3, r1, r2

| Field | funct7 | rs2 | rs1 | funct3 | rd | opcode |
| --- | --- | --- | --- | --- | --- | --- |
| Name | — | r2 | r1 | ADD | r3 | R-type |
| Binary | 0000000 | 00010 | 00001 | 000 | 00011 | 1001101 |

Full 32-bit binary:

0000000 00010 00001 000 00011 1001101
= 00000000001000001000000111001101

Hex: `0x002081CD`

#### I-type: kopanya_hang r5, r1, 10

| Field | imm[11:0] | rs1 | funct3 | rd | opcode |
| --- | --- | --- | --- | --- | --- |
| Name | 10 | r1 | ADDI | r5 | I-type |
| Binary | 000000001010 | 00001 | 100 | 00101 | 1001001 |

Full 32-bit binary:

000000001010 00001 100 00101 1001001
= 00000000101000001100001011001001

Hex: `0x00A0C2C9`

#### S-type: boloka r3, 8(r2)

Offset 8 = 000000001000 (12-bit). imm[11:5] = 0000000, imm[4:0] = 01000.

| Field | imm[11:5] | rs2 | rs1 | funct3 | imm[4:0] | opcode |
| --- | --- | --- | --- | --- | --- | --- |
| Name | 8[11:5] | r3 | r2 | SW | 8[4:0] | S-type |
| Binary | 0000000 | 00011 | 00010 | 000 | 01000 | 0101101 |

Full 32-bit binary:

0000000 00011 00010 000 01000 0101101
= 00000000001100010000010000101101

Hex: `0x0031042D`

#### B-type: lekana r1, r2, 8

Offset +8 as 13-bit: bit[12]=0, bit[11]=0, bits[10:5]=000000, bits[4:1]=0100.

| Field | imm[12] | imm[10:5] | rs2 | rs1 | funct3 | imm[4:1] | imm[11] | opcode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name | 8[12] | 8[10:5] | r2 | r1 | BEQ | 8[4:1] | 8[11] | B-type |
| Binary | 0 | 000000 | 00010 | 00001 | 000 | 0100 | 0 | 1001110 |

Full 32-bit binary:

0 000000 00010 00001 000 0100 0 1001110
= 00000000010000001000010001001110

Hex: `0x0020844E`

## A6. Design-Decision Log

(Each answer is at most 120 words, with evidence from our own programs where specified.)

### (a) Register-file size: cost and benefit

Choosing 32 registers means each register field is 5 bits wide. An R-type instruction spends 15 of its 32 bits on register addresses, leaving 17 bits for opcode and funct fields — identical to RV32I and sufficient for our 12-instruction set. The forwarding unit requires four 5-bit comparators to detect RAW hazards across EX/MEM and MEM/WB stages. The benefit is low register pressure: in our sample programs, all intermediate values remain in registers throughout computation with no spills to data memory, keeping instruction counts low. The cost is those 15 encoding bits — a 16-register design would reclaim 3 bits, usable for wider immediates, but at the expense of increased register spilling in loops.

### (b) Architectural vs microarchitectural registers

LIMO's architectural registers — the 32 GPRs, PC (sebali_tsamaiso), zero register (noto), stack pointer (ts'upiso_phaello) and return address (khutla_aterese) — are programmer-visible and fully specified in the ISA. The microarchitectural registers — IF/ID, ID/EX, EX/MEM and MEM/WB pipeline registers and the instruction register — exist only to carry data between pipeline stages and are invisible to software. The ISA must not specify them because they are an implementation detail: a valid LIMO implementation could use a different pipeline depth or out-of-order execution and still honour the ISA contract. Exposing pipeline registers in the ISA would permanently bind every implementation to one specific microarchitecture, eliminating the implementation freedom that separating ISA from microarchitecture exists to provide.

### (c) Branch resolution stage and flush cost

LIMO resolves branches in the EX stage, where the ALU computes the comparison and the branch target is available. By EX, two instructions have already entered the pipeline behind the branch — one in IF and one in ID — so both must be flushed on a taken branch, costing 2 cycles per taken branch. Resolving in ID would reduce flushes to 1 but requires a dedicated comparator and branch-target adder in the decode stage, adding significant hardware complexity. Given LIMO's small instruction set and simulator implementation goals, EX resolution offers the better complexity tradeoff. Our sample program 3 (counting loop) shows taken branches are infrequent enough that the 2-cycle penalty has minimal overall CPI impact.

### (d) Load-use stall under full forwarding

A load's result is not available until the end of the MEM stage. A dependent instruction needs that value at the start of its EX stage. Even with full forwarding, the value would need to travel backwards in time — from MEM of the load to EX of the dependent instruction in the same cycle. This is physically impossible; one stall cycle is mandatory.

C1    C2    C3    C4    C5    C6
bala     IF    ID    EX   MEM    WB
kopanya        IF    ID   ***    EX   MEM   WB
                           ↑
                        stall inserted here
                        value exits MEM at end of C4
                        kopanya needs it at start of EX
                        forwarded from MEM/WB to EX in C5

The bubble (***) is inserted by the hazard unit, which stalls kopanya for one cycle until the load result can be forwarded from MEM/WB.

### (e) Flags register: would LIMO benefit?

LIMO's branches (lekana, kholo_ho, hali_lekane) embed their comparison directly into the branch instruction, following the RISC-V model. A flags register would separate comparison from branching: a compare instruction sets flags, a branch reads them. This adds expressive flexibility but creates a new RAW hazard: every branch becomes dependent on the preceding compare, requiring the forwarding unit to track the flags register as additional architectural state. Since LIMO has no multiply or complex operations that naturally produce flag outputs, the added hazard complexity outweighs the benefit. Keeping comparisons inside branch instructions means one instruction, one pipeline pass, and no flag-forwarding logic — simpler hardware and no extra stall cases in the hazard unit.

### (f) What breaks first as programs grow

With 32 registers, register pressure is the last concern — LIMO programs can hold many live values simultaneously without memory spills. Immediate range (12-bit signed, −2048 to +2047) limits large constants and array offsets but only affects specific access patterns. Branch reach breaks first. LIMO's B-type immediate gives a signed 13-bit offset, reaching ±4 KB — roughly ±1000 instructions. As programs grow beyond this, forward and backward branches to distant labels exceed the encodable offset. LIMO currently has no J-type jump or indirect-branch instruction, so it cannot express a general workaround for an out-of-range branch using the instructions defined here. The practical fix in a future ISA revision would be to add a J-type jump (or another explicitly specified long-range control-transfer instruction). Until then, the assembler must report an out-of-range branch rather than silently emit an unsupported sequence. This is the first structural limitation a larger LIMO program would encounter.

## Sample Programs

All programs are written in LIMO assembly and may be assembled and run on the LIMO simulator. Expected register and memory states after execution are given for each program.

### Program 1: Arithmetic and Logic

Computes result = (a + b) AND (a − b) where a = 12, b = 5.

# LIMO Program 1 — Arithmetic and Logic
# Computes: r5 = (r1 + r2) AND (r1 - r2)
# Expected results: r1=12, r2=5, r3=17, r4=7, r5=1

kopanya_hang r1, r0, 12     # r1 = 12           (a)
kopanya_hang r2, r0, 5      # r2 = 5            (b)
kopanya      r3, r1, r2     # r3 = 12 + 5 = 17  (sum)
tlosa        r4, r1, r2     # r4 = 12 - 5 = 7   (difference)
le           r5, r3, r4     # r5 = 17 AND 7 = 1 (bitwise AND)
#
# Verification:
#   17 = 10001
#    7 = 00111
#  AND = 00001 = 1 ✓

Expected final register state:

| Register | Value |
| --- | --- |
| r1 | 12 |
| r2 | 5 |
| r3 | 17 |
| r4 | 7 |
| r5 | 1 |

Instruction classes exercised: arithmetic (kopanya, tlosa, kopanya_hang), logic (le).

### Program 2: Memory Load and Store

Loads a word from memory, doubles it by adding it to itself, and stores the result at the next word address.

# LIMO Program 2 — Memory Access
# Assumes Memory[100] = 21 is pre-loaded before execution
# Expected: Memory[100] = 21 (unchanged), Memory[104] = 42

kopanya_hang r1, r0, 100    # r1 = 100           (base address)
bala         r2, 0(r1)      # r2 = Memory[100] = 21
kopanya      r3, r2, r2     # r3 = 21 + 21 = 42  (double r2)
boloka       r3, 4(r1)      # Memory[104] = 42

Expected final register and memory state:

| Register | Value |
| --- | --- |
| r1 | 100 |
| r2 | 21 |
| r3 | 42 |

| Address | Value |
| --- | --- |
| 100 | 21 |
| 104 | 42 |

Instruction classes exercised: arithmetic immediate (kopanya_hang), arithmetic (kopanya), memory load (bala), memory store (boloka).

Hazard present: kopanya r3, r2, r2 immediately follows bala r2, 0(r1) — this is a load-use hazard. The simulator must insert one stall bubble before the kopanya instruction proceeds to EX. This program demonstrates A6(d) in practice.

### Program 3: Conditional Loop (Accumulator)

Computes the sum 5 + 4 + 3 + 2 + 1 = 15 using a countdown loop.

# LIMO Program 3 — Conditional Branch and Loop
# Computes: r5 = 5 + 4 + 3 + 2 + 1 = 15
# r1 = loop counter (5 down to 1)
# r2 = decrement (constant 1)
# r3 = loop exit threshold (constant 1)
# r5 = accumulator

kopanya_hang r1, r0, 5      # r1 = 5  (counter, starts at 5)
kopanya_hang r2, r0, 1      # r2 = 1  (decrement value)
kopanya_hang r3, r0, 1      # r3 = 1  (loop exit threshold)
kopanya_hang r5, r0, 0      # r5 = 0  (accumulator)

boelela:
    kopanya r5, r5, r1      # r5 = r5 + r1  (accumulate counter)
    tlosa   r1, r1, r2      # r1 = r1 - 1   (decrement counter)
    kholo_ho r1, r3, boelela # if r1 >= 1, branch back to boelela

# Loop terminates when r1 = 0, at which point r1 >= 1 is false
# r5 = 5 + 4 + 3 + 2 + 1 = 15

Expected final register state:

| Register | Value | Notes |
| --- | --- | --- |
| r1 | 0 | counter exhausted |
| r2 | 1 | unchanged |
| r3 | 1 | unchanged |
| r5 | 15 | = 5+4+3+2+1 |

Cycle trace (loop body only, iterations 1–5):

| Iteration | r1 before | Added to r5 | r1 after | Branch taken? |
| --- | --- | --- | --- | --- |
| 1 | 5 | 5 | 4 | Yes |
| 2 | 4 | 4 | 3 | Yes |
| 3 | 3 | 3 | 2 | Yes |
| 4 | 2 | 2 | 1 | Yes |
| 5 | 1 | 1 | 0 | No (exit) |

Instruction classes exercised: arithmetic immediate (kopanya_hang), arithmetic (kopanya, tlosa), conditional branch (kholo_ho). This program also exercises the branch flush mechanism: each taken branch in the loop body flushes 2 instructions from the pipeline (A6(c)), visible in the simulator's pipeline chart as bubbles on the cycle following each taken kholo_ho.

Label: boelela — Sesotho for "go back / repeat".

End of LIMO ISA Specification — M1